Make your own Linux Distro From scratch

I'll call my distro name as: TiniLinux

Welcome to TiniLinux, we will make a Linux Distro from scratch

Theory

To make a very tiny and simple Linux distro, we're going to need:

Kernel

User Space (in this case we use BusyBox)

Bootloader (in this case we use Grub)

┌────────────────────┐

│     Power On       │

│                    │

└─────────┬──────────┘

│

│

┌─────────▼──────────┐

│     BIOS/UEFI      │

│                    │

│       (POST)       │

└─────────┬──────────┘

│

│

┌─────────▼──────────┐

│    Boot Device     │

│                    │

│  Hard Disk/CD/USB  │

└─────────┬──────────┘

│

│

┌─────────▼──────────┐

│  Boot Loader(GRUB) │

│                    │

│  Load kernel and   │

│  initramfs to RAM  │

└──────────┬─────────┘

│

│

┌──────────▼─────────┐

│       Kernel       │

│                    │

│ Execute /sbin/init │

└──────────┬─────────┘

│

(Live boot)   │  (Boot from Hard disk)

┌─────────────┴──────────────┐

│                            │

┌──────────▼─────────┐       ┌──────────▼─────────┐

│   Init (initramfs) │       │  Init (initramfs)  │

│                    │       │  Mount Real rootfs │

│   Execute RunLevel │       │  from hard disk    │

│      programs      │       │  then switch_root  │

│                    │       │  exec /sbin/init   │

└────────────────────┘       └──────────┬─────────┘

│

│

┌──────────▼─────────┐

│        Init        │

│                    │

│   Execute RunLevel │

│      programs      │

└──────────┬─────────┘

│

│

┌──────────▼─────────┐

│  Systemd / OpenRC  │

│                    │

│   Execute System   │

│   Daemon programs  │

└────────────────────┘

(Drew with https://asciiflow.com/)

This applies to real world Linux distro as well (Debian, Ubuntu, Fedora,...)

1. Setting up the environment:

Update the package list and install necessary tools:

sudo apt update

sudo apt install build-essential git flex bison bc cpio libncurses5-dev libssl-dev libelf-dev

export WORKDIR=/home/${USER}/TiniLinux

mkdir -p ${WORKDIR}/source                        # for linux kernel & busybox source code

mkdir -p ${WORKDIR}/build/amd64/initramfs         # for built kernel & busybox

mkdir -p ${WORKDIR}/build/amd64/uefi/boot/grub    # for making boot iso image

2. Compile the kernel:

Clone the Linux kernel source code:

cd ${WORKDIR}/source

git clone --depth=1 https://github.com/torvalds/linux.git

cd linux

make defconfig

make menuconfig

# Exit then save. The default config is good enough. (but nothing is stopping you from messing around it!)

# NOTE: Fix Black Screen After Grub When Booting in UEFI

# When booting in UEFI, after choosing your OS in grub, your screen might go blank and stay blank. This might indicate a problem in your kernel config

# make menuconfig -> press "/" button to search for config

# Search for "FB_EFI" -> Press "1" (the small number on the left of search result to quickly go to the config) -> Press "y" -> "Esc Esc" -> "1" -> "y"

# Search for "FRAMEBUFFER_CONSOLE" -> Press 1 -> Press "y"

# Some other recommended config to turn on:

# IKCONFIG, IKCONFIG_PROC: read kernel .config through /proc/config.gz

# EFIVAR_FS: mount EFI vars

# OVERLAY_FS, BRIDGE, BRIDGE_NETFILTER, IP_NF_NAT, IP_NF_TARGET_MASQUERADE, VETH, IP_VS, NETFILTER_XT_MATCH_ADDRTYPE, NETFILTER_XT_MATCH_IPVS... (check more in docker section): to enable docker

# HYPERV, HYPERV_KEYBOARD, HYPERV_NET, HYPERV_STORAGE, HYPERV_UTILS, (INPUT_MOUSEDEV, HID_HYPERV_MOUSE if enable GUI)

# Save

# to compile the kernel with all of your cores

make -j $(nproc)

cp arch/x86/boot/bzImage ${WORKDIR}/build/amd64/uefi/boot/

# cp arch/x86/boot/bzImage ${WORKDIR}/build/amd64/bios/boot/

The compiled kernel image is copied to a directory named bootfiles.

Kernel modules

bzImage is the core of the kernel, which will be loaded to the memory from system boot to shutdown. Linux kernel support many features and drivers, you may not want to add them as built-in [*], instead make them modules [M], these can be loaded or unloaded after boot as hot plug when needed

# Copy compiled modules to a directory

make modules_install INSTALL_MOD_PATH=${WORKDIR}/build/amd64/kernel-modules/

3. Compile Busybox:

BusyBox combines tiny versions of many common UNIX utilities into a single small executable.

cd ${WORKDIR}/source

git clone --depth=1 https://git.busybox.net/busybox

cd busybox

make menuconfig

# Now, enter the first sub-menu (Settings). Go down until you reach the section Build options. Select the Build static binary (no shared libs)

# Optional: Go back to main menu. Scroll down until you find the Shells section. Two first options should be Choose which shell is aliased to 'sh' name and Choose which shell is aliased to 'bash' name. Choose each and tell BusyBox to alias sh to Ash and bash to Hush. You might also want to tweak some options but they are not required

# Exit then save

make -j $(nproc)

make CONFIG_PREFIX=${WORKDIR}/build/amd64/initramfs install

The built Busybox binaries are installed into the build/amd64/initramfs directory. Now create a init script within initramfs to launch Busybox upon boot:

cd ${WORKDIR}/build/amd64/initramfs

cat <<EOF > init

#!/bin/sh

/bin/sh

EOF

chmod +x init

3.1. Advance (advanced init and login)

cd ${WORKDIR}/build/amd64/initramfs

# create some directories to give it a minimal Linux-like look

mkdir -p {etc,dev,mnt,root,sys,proc,dev,run,tmp,var,home,etc/init.d}

# /etc : We need to fill in a couple of files here

cat <<EOF > etc/inittab

# https://github.com/brgl/busybox/blob/master/examples/inittab

::sysinit:/etc/init.d/rcS

tty1::respawn:/sbin/getty 0 tty1

tty2::respawn:/sbin/getty 0 tty2

tty3::respawn:/sbin/getty 0 tty3

tty4::respawn:/sbin/getty 0 tty4

tty5::respawn:/sbin/getty 0 tty5

tty6::respawn:/sbin/getty 0 tty6

# Enable these ttyS if you want to use serial (qemu -serial stdio)

#ttyS0::respawn:/sbin/getty -L ttyS0 9600 vt100

::restart:/sbin/init

::ctrlaltdel:/sbin/reboot

::shutdown:/bin/umount -a -r

::shutdown:/sbin/swapoff -a

EOF

cat <<\EOF > etc/init.d/rcS

#!/bin/sh

mount -t sysfs    sysfs     -o nosuid,noexec,nodev    /sys

mount -t proc     proc      -o nosuid,noexec,nodev    /proc

mount -t devtmpfs devtmpfs  -o nosuid,mode=0755       /dev

mount -t tmpfs    tmpfs     -o nosuid,nodev,mode=755  /run

mkdir -p /dev/pts

mount -t devpts   devpts    -o nosuid,gid=5,mode=620  /dev/pts

mount -a

mdev -s

hostname tinilinux

ip link set lo up

echo 5 > /proc/sys/kernel/printk

syslogd

cat <<!

Boot took $(cut -d' ' -f1 /proc/uptime) seconds

_____ _       _ _     _

|_   _(_)_ __ (_) |   (_)_ __  _   ___  __

| | | | '_ \| | |   | | '_ \| | | \ \/ /

| | | | | | | | |___| | | | | |_| |>  <

|_| |_|_| |_|_|_____|_|_| |_|\__,_/_/\_\\

Welcome to TiniLinux

!

EOF

chmod +x etc/init.d/rcS

cat <<EOF > etc/hostname

tinilinux

EOF

cat <<EOF > etc/passwd

root::0:0:root:/root:/bin/bash

EOF

cat <<EOF > etc/group

root:x:0:root

EOF

cat <<EOF > etc/fstab

# https://www.linuxfromscratch.org/lfs/view/10.0/chapter10/fstab.html

# file system  mount-point  type     options                dump   fsck order

# sysfs          /sys         sysfs    nosuid,noexec,nodev    0      0

# proc           /proc        proc     nosuid,noexec,nodev    0      0

# devtmpfs       /dev         devtmpfs nosuid,mode=0755       0      0

# tmpfs          /run         tmpfs    nosuid,nodev,mode=755  0      0

# devpts         /dev/pts     devpts   nosuid,gid=5,mode=620  0      0

# /dev/sda2      /            ext4     defaults               0      1

# /dev/sda1      /boot/efi    vfat     umask=0077             0      1

EOF

cat <<EOF > init

#!/bin/sh

exec /linuxrc

EOF

chmod +x init

3.2. Advance (network)

mkdir -p etc/network/if-up.d

mkdir -p etc/network/if-pre-up.d

mkdir -p etc/network/if-down.d

mkdir -p etc/network/if-post-down.d

mkdir -p var/run

mkdir -p usr/share/udhcpc

cat <<EOF > etc/hosts

127.0.0.1       localhost

127.0.1.1       tinilinux

EOF

cat <<EOF > etc/network/interfaces

# enable dhcp for eth0 interface: https://wiki.alpinelinux.org/wiki/Udhcpc

auto eth0

iface eth0 inet dhcp

hostname tinilinux

EOF

cat <<\EOF > usr/share/udhcpc/default.script

#!/bin/ash

# https://github.com/mschlenker/TinyCrossLinux/blob/master/patches/usr-share-udhcpc-default.script

PATH=/bin:/sbin:/usr/bin:/usr/sbin

export PATH

[ -z "$1" ] && echo "Error: should be called from udhcpc" && exit 1

RESOLV_CONF="/etc/resolv.conf"

[ -n "$broadcast" ] && BROADCAST="broadcast $broadcast"

[ -n "$subnet" ] && NETMASK="netmask $subnet"

case "$1" in

deconfig)

ifconfig $interface 0.0.0.0

;;

renew|bound)

ifconfig $interface $ip $BROADCAST $NETMASK

if [ -n "$router" ] ; then

echo "deleting routers"

while route del default gw 0.0.0.0 dev $interface ; do

:

done

for i in $router ; do

route add default gw $i dev $interface

done

fi

echo -n > $RESOLV_CONF

[ -n "$domain" ] && echo search $domain >> $RESOLV_CONF

for i in $dns ; do

echo adding dns $i

echo nameserver $i >> $RESOLV_CONF

done

;;

esac

exit 0

EOF

chmod +x usr/share/udhcpc/default.script

# add line "ifup eth0" after "ip link set lo up":

sed -i 's/ip link set lo up/ip link set lo up\nifup eth0/g' etc/init.d/rcS

3.3. SSH (dropbear)

Build dropbear binary from source

cd ${WORKDIR}/source

git clone --depth=1 https://github.com/mkj/dropbear.git

cd dropbear

./configure --enable-static

cp src/default_options.h localoptions.h # update options if you want

make PROGRAMS="dropbear dbclient dropbearkey dropbearconvert scp" MULTI=1 STATIC=1 strip

Copy dropbear binary to initramfs

cp ${WORKDIR}/source/dropbear/dropbearmulti ${WORKDIR}/build/amd64/initramfs/usr/bin/

cd ${WORKDIR}/build/amd64/initramfs/usr/bin/

ln -s dropbearmulti scp

ln -s dropbearmulti dropbearkey

ln -s dropbearmulti dropbearconvert

ln -s dropbearmulti dropbear

ln -s dropbearmulti ssh

ln -s dropbearmulti dbclien

cd ${WORKDIR}/build/amd64/initramfs/

mkdir -p {var/log,etc/default,etc/dropbear}

touch var/log/lastlog

cat <<EOF > etc/shells

/bin/bash

/bin/sh

EOF

After boot, start dropbear with

dropbear -EBR # log error on std error, allow blank password, create hostkey if required

or add to rcS

sed -i 's/ifup eth0/ifup eth0 && dropbear -RB/g' etc/init.d/rcS

3.4. curl

I prefer curl over wget, so if you want to use curl

# check version and arch here: https://github.com/stunnel/static-curl/releases

curl -LO https://github.com/stunnel/static-curl/releases/download/8.6.0-1/curl-linux-x86_64-8.6.0.tar.xz

tar -xvf curl-linux-x86_64-8.6.0.tar.xz

rm curl-linux-x86_64-8.6.0.tar.xz

# you may want to compress the binary: $ upx --brute curl

mv curl usr/bin

4. Creating the initial ram filesystem (initramfs):

The find command is used to create a list of files within initramfs, and the cpio command archives them into a compressed image named initramfs.cpio.

cd ${WORKDIR}/build/amd64/initramfs/

sudo chown -R root:root *

find . | cpio -o -H newc | gzip -9 > ${WORKDIR}/build/amd64/uefi/boot/initramfs.cpio.gz

sudo chown -R $USER:$(id -gn) *

Test the built kernel and initramfs

sudo apt install qemu-system

qemu-system-x86_64 -kernel bzImage -initrd initramfs.cpio.gz

qemu-system-x86_64 -nographic -append console=ttyS0 -kernel bzImage -initrd initramfs.cpio.gz

5. Configuring the bootloader:

5.1. Use grub (recommend)

We will use grub-mkrescue to create our bootable ISO. But before doing so, we have to know if our current host is booted with UEFI or BIOS. To do so, check if the folder /sys/firmware/efi exists on your system or not. If it does, your computer uses UEFI otherwise it’s BIOS.

So why knowing this is important? The grub-mkrescue uses the currently installed grub stuff to create the ISO image. This means that if your operating system is booted in BIOS, the chances are that the ISO created from grub-mkrescue does not support UEFI at all. In some cases, UEFI motherboards support booting BIOS images using CMS. But that’s not always the case. If you want to make images for BIOS from UEFI host or vice versa, I suggest you to create a Debian virtual machine in VirtualBox. VirtualBox supports both BIOS and UEFI in it’s motherboard settings. After choosing the appropriate one, install Debian (net install is sufficient) and move the folder which contains boot and grub folders to virtual machine

If you use grub in wsl2: https://github.com/Microsoft/WSL/issues/807#issuecomment-356697269

Create a folder somewhere with any name you want. I name it iso. Then create a folder called boot in it and inside boot create a folder called grub. Then copy bzImage and initramfs.cpio.gz into boot folder.

sudo apt install xorriso mtools

cd ${WORKDIR}/build/amd64/

UEFI

cat <<EOF > uefi/boot/grub/grub.cfg

set default=0

set timeout=2

# Load EFI video drivers. This device is EFI so keep the video mode while booting the linux kernel.

insmod efi_gop

insmod font

if loadfont /boot/grub/fonts/unicode.pf2

then

insmod gfxterm

set gfxmode=auto

set gfxpayload=keep

terminal_output gfxterm

fi

menuentry 'TiniLinux' --class os {

insmod gzio

insmod part_msdos

linux /boot/bzImage

initrd /boot/initramfs.cpio.gz

}

EOF

grub-mkrescue -o tinilinux-amd64-uefi.iso uefi

BIOS

cat <<EOF > bios/boot/grub/grub.cfg

set default=0

set timeout=2

menuentry 'TiniLinux' --class os {

insmod gzio

insmod part_msdos

linux /boot/bzImage

initrd /boot/initramfs.cpio.gz

}

EOF

grub-mkrescue -o tinilinux-bios-uefi.iso bios/

Note: uefi iso can boot with both uefi and bios

5.2. Use syslinux

The syslinux package is installed to provide the bootloader. A blank disk image is created and formatted with a FAT filesystem. The kernel image and initramfs are copied to the disk image.

# using syslinux

sudo apt install syslinux dosfstools

dd if=/dev/zero of=boot bs=1M count=50

mkfs -t fat boot

syslinux boot

mkdir m

sudo mount boot m

cp bzImage init.cpio m

umount m

Test:

qemu-system-x86_64 boot

Booting the distro:

The iso file now can be used with VirtualBox or VMWare or burn to USB/CD

From here, the basic steps of making a Linux distro from scatch are complete

Install a package manager (apk)

We will use apk (Alpine package manager) from Alpine Linux because it's very lightweight and designed to run on RAM (as Alpine Linux)

Pre-install in the iso file

cd ${WORKDIR}/build/amd64/initramfs

wget https://gitlab.alpinelinux.org/api/v4/projects/5/packages/generic//v2.14.0/x86_64/apk.static

sudo chroot .

cp /bin/busybox /bin/busybox-bak # backup original busybox as apk add will replace it

cat <<EOF > /etc/resolv.conf

nameserver 8.8.8.8

EOF

mv apk.static apk

chmod +x apk

mv apk /usr/bin/apk

mkdir -p /lib/apk/db/

mkdir -p /etc/apk/

touch /etc/apk/world

cat <<EOF > /etc/apk/repositories

http://dl-cdn.alpinelinux.org/alpine/v3.19/main

http://dl-cdn.alpinelinux.org/alpine/v3.19/community

EOF

apk add --allow-untrusted --no-cache ca-certificates alpine-keys

cat <<EOF > /etc/apk/repositories

https://dl-cdn.alpinelinux.org/alpine/v3.19/main

https://dl-cdn.alpinelinux.org/alpine/v3.19/community

EOF

mv -f /bin/busybox-bak /bin/busybox

rm -rf /lib/apk/exec

rm -rf /lib/apk/db/lock

rm -rf /var/cache/apk/*

exit

sudo rm -rf root/.bash_history

sudo rm -rf root/.hush_history

Use apk

After booted from the iso, we can use apk normally

apk update

apk add bash wget coreutils

apk add python3

Alpine Linux utilities

Since we're using apk, it comes with a lot of useful utilities thanks to Alpine Linux

apk add alpine-conf

# https://wiki.alpinelinux.org/wiki/Alpine_setup_scripts

setup-sshd

setup-desktop

Install to hard drive¶

At this stage, everything is basically complete, you have a complete live boot ISO. You may want to install it to hard drive and use as or normal Operating System.

Make an early boot initramfs¶

First, we have to have a new fresh early-boot initramfs. This early-boot initramfs is different from live boot initramfs as after installed to hard disk, when you boot, the bootloader will load the kernel and this early-boot initramfs, it will mount the hard drive (the main root filesystem) to /mnt/.root, switch_root to it, and then call /sbin/init again. After that the system will be init from root file system from the hard disk

Boot Loader -> Load Kernel + Load initramfs -> switch_root to rootfs from hard disk

## Update initramfs for boot from hard drive, from your main PC:

# if not yet have the initramfs-boot

# cd ${WORKDIR}/source/busybox/

# make CONFIG_PREFIX=${WORKDIR}/build/amd64/initramfs-boot install

cd ${WORKDIR}/build/amd64/initramfs-boot

mkdir dev proc sys run

cat <<\EOF > init

#!/bin/sh

ROOT="/mnt/.root"

ROOT_DEV="/dev/sda2"

echo "init from initramfs"

# mount temporary filesystems

mount -n -t devtmpfs devtmpfs /dev

mount -n -t proc     proc     /proc

mount -n -t sysfs    sysfs    /sys

mount -n -t tmpfs    tmpfs    /run

# mount new root

[ -d ${ROOT} ] || mkdir -p ${ROOT}

mount ${ROOT_DEV} ${ROOT}

# switch to new rootfs and exec init

cd ${ROOT}

exec switch_root . "/sbin/init" "$@"

EOF

chmod +x init

# rebuild initramfs

mkdir ../initramfs/boot/

sudo chown -R root:root * && find . | cpio -o -H newc | gzip -9 > ../initramfs/boot/initramfs.cpio.gz && sudo chown -R $USER:$(id -gn) *

cd ${WORKDIR}/build/amd64/initramfs

cp ${WORKDIR}/source/linux/arch/x86/boot/bzImage ${WORKDIR}/build/amd64/initramfs/boot/

cp -r ${WORKDIR}/build/amd64/kernel-modules/lib/modules ${WORKDIR}/build/amd64/initramfs/lib

### Build the iso again and start booting from it, proceed to the next step

cd ${WORKDIR}/build/amd64/initramfs

sudo chown -R root:root * && find . | cpio -o -H newc | gzip -9 > ${WORKDIR}/build/amd64/uefi/boot/initramfs.cpio.gz && sudo chown -R $USER:$(id -gn) *

grub-mkrescue -o ${WORKDIR}/build/amd64/tinilinux-amd64-uefi.iso ${WORKDIR}/build/amd64/uefi

Start installing to hard drive¶

Now boot using the iso, after booted from the iso:

apk add --allow-untrusted --no-cache ca-certificates alpine-keys

apk add --no-cache bash wget lsblk cfdisk grub grub-bios grub-efi dosfstools e2fsprogs rsync efibootmgr

cat <<EOF > /etc/group

root:x:0:

daemon:x:1:

bin:x:2:

sys:x:3:

adm:x:4:syslog

tty:x:5:

disk:x:6:

lp:x:7:

mail:x:8:

news:x:9:

uucp:x:10:

man:x:12:

proxy:x:13:

kmem:x:15:

dialout:x:20:

fax:x:21:

voice:x:22:

cdrom:x:24:

floppy:x:25:

tape:x:26:

sudo:x:27:

audio:x:29:

dip:x:30:

www-data:x:33:

backup:x:34:

operator:x:37:

list:x:38:

irc:x:39:

src:x:40:

gnats:x:41:

shadow:x:42:

utmp:x:43:

video:x:44:

sasl:x:45:

plugdev:x:46:

staff:x:50:

games:x:60:

users:x:100:

nogroup:x:65534:

EOF

# create partition

# /dev/sda1: for boot   (must be vfat or fat32), 100MB is ok

# /dev/sda2: for rootfs (can be ext4), the rest of disk storage

cfdisk

# choose GPT for UEFI or dos for BIOS boot

# type can be default: Linux filesystem

# remember to [Write] all /dev/sda* before [Quit]

mdev -s

# format partition with the correct type

mkfs.fat -F32 /dev/sda1

mkfs.ext4 /dev/sda2

# mount disk and install grub

mount /dev/sda2 /mnt

## use rsync to copy the entire root filesystem to another location

rsync -aAXv /* /mnt --exclude={/dev/*,/proc/*,/sys/*,/tmp/*,/run/*,/mnt/*,/media/*,/lost+found}

mkdir -p /mnt/boot/efi

mount /dev/sda1 /mnt/boot/efi

mount --bind /dev/ /mnt/dev/

mount --bind /proc/ /mnt/proc/

mount --bind /sys/ /mnt/sys/

chroot /mnt

mount -t efivarfs efivarfs /sys/firmware/efi/efivars

grub-install --target=x86_64-efi --efi-directory=/boot/efi --bootloader-id=TiniLinux-UEFI #--bootloader-id=GRUB --no-nvram --removable

# then add this line to /etc/default/grub

cat <<\EOF > /etc/default/grub

GRUB_DISTRIBUTOR="TiniLinux"

GRUB_TIMEOUT=2

GRUB_DISABLE_SUBMENU=y

GRUB_DISABLE_RECOVERY=true

GRUB_CMDLINE_LINUX_DEFAULT="quiet"

GRUB_CMDLINE_LINUX=""

EOF

#check disk id: blkid

#/dev/sda2: UUID="4c8d1658-283e-4a07-9ab6-7efd4623e094" TYPE="ext4"

#/dev/sda1: UUID="8CE4-73DF" TYPE="vfat"

# then add this line to /etc/grub.d/40_custom

cat <<\EOF >> /etc/grub.d/40_custom

menuentry "TiniLinux" {

set root=(hd0,gpt2)

echo "Loading kernel..."

linux /boot/bzImage root=/dev/sda2 ro quiet splash

echo "Loading initramfs..."

initrd /boot/initramfs.cpio.gz

}

EOF

mv /etc/grub.d/30_uefi-firmware /etc/grub.d/50_uefi-firmware

update-grub

# check grub.cfg for the added menuentry

cat /boot/grub/grub.cfg

exit

umount /mnt/proc /mnt/sys/firmware/efi/efivars /mnt/sys /mnt/dev

mkdir -p /mnt/dev/pts

umount /dev/sda1

umount /dev/sda2

exit

Remove live boot CD or USB Now try boot from hard disk if it fail, from grub menu press e to edit the entry or press c to enter command line

ls

set root=(hd0,gpt2)

linux /boot/bzImage root=/dev/sda2

initrd /boot/initramfs.cpio.gz

boot

Now we can boot, after booted, run update-grub again Done

Post installation

After installation, you can do these steps to make your OS more Linux-like OS

rdD

Contrary with rcS, this script will be excecuted when the system shutdown or restart

# add this line before any ::shutdown lines in /etc/inittab

::shutdown:/etc/rcD

cat <<\EOF >> /etc/rcD

#!/bin/sh

ifdown eth0

EOF

chmod +x /etc/rcD

Enable syslogd

# Add to /etc/init.d/rcS

syslogd

# Now you can view system log

cat /var/log/messages

Users

Create normal users

apk add shadow

touch /etc/shadow

chmod 640 /etc/shadow

addgroup haoict

adduser haoict -G haoict

addgroup -g 27 sudo

apk add sudo

vi /etc/sudoers

# uncomment %sudo   ALL=(ALL:ALL) ALL

addgroup haoict sudo

sudo chmod 666 /dev/urandom

sudo chmod 666 /dev/null

sudo chmod -R 777 /tmp

Disable ipv6

echo 0 > /proc/sys/net/ipv6/conf/all/autoconf

echo 0 > /proc/sys/net/ipv6/conf/all/accept_ra

echo 1 > /proc/sys/net/ipv6/conf/all/disable_ipv6

Patch the kernel

Sometimes you may want to add some features or driver modules, you can compile the kernel again and copy it to /boot/ and /lib/modules

cd ${WORKDIR}/source/linux

make menuconfig

make -j $(nproc)

make modules_install INSTALL_MOD_PATH=${WORKDIR}/build/amd64/kernel-modules/

rsync -lr ${WORKDIR}/build/amd64/kernel-modules/lib/modules root@192.168.1.131:/lib/

scp ${WORKDIR}/source/linux/arch/x86/boot/bzImage root@192.168.1.131:/boot/

# to load kernel module

modprobe <module_name>

# list loadable modules

find /lib/modules/$(uname -r) -type f -name '*.ko'

cat /lib/modules/$(uname -r)/modules.alias #  (with alias)

# list loaded modules

lsmod

Cross compilation (arm64)¶

sudo apt-get install gcc-aarch64-linux-gnu g++-aarch64-linux-gnu

#make ARCH=arm64 CROSS_COMPILE=aarch64-linux-gnu- defconfig

make ARCH=arm64 CROSS_COMPILE=aarch64-linux-gnu- menuconfig

make ARCH=arm64 CROSS_COMPILE=aarch64-linux-gnu- -j 8

make ARCH=arm64 CROSS_COMPILE=aarch64-linux-gnu- CONFIG_PREFIX=${WORKDIR}/build/arm64/initramfs install

# follow the same steps to create initramfs.cpio.gz

cd ${WORKDIR}/build/arm64/initramfs

sudo chown -R root:root * && find . | cpio -o -H newc | gzip -9 > ${WORKDIR}/build/arm64/uefi/boot/initramfs.cpio.gz && sudo chown -R $USER:$(id -gn) *

# test

## ref: https://gist.github.com/billti/d904fd6124bf6f10ba2c1e3736f0f0f7

## list cpu: qemu-system-aarch64 -M virt -cpu help

qemu-system-aarch64 -m 2048 -cpu cortex-a72 -smp 4 -M virt -nographic -kernel Image -initrd initramfs.cpio.gz

Make the iso and boot?? TBD (I haven't figured out yet, beside a Raspberry Pi which is running well and I don't want to mess with it, I don't have any spare arm64 board)

Install GUI (X11)¶

apk add alpine-conf

# setup mdev: https://wiki.alpinelinux.org/wiki/Mdev

setup-devd udev # or apk add busybox-mdev-openrc

setup-desktop xfce

rc-update del lightdm

rc-status # check openrc

apk add xterm

apk add xf86-video-vmware xf86-video-fbdev xf86-video-vesa

/sbin/openrc sysinit

/sbin/openrc boot

startx # or startxfce4 or xinit xfce4-session

# for gnome: xinit gnome-session

# for mate: xinit mate-session

# for plasma: xinit plasma_session

# extra

apk add open-vm-tools xf86-input-vmmouse

/etc/init.d/open-vm-tools start

startx

All in one command for built & test¶

cd ${WORKDIR}/build/amd64/initramfs

sudo chown -R root:root * && find . | cpio -o -H newc | gzip -9 > ${WORKDIR}/build/amd64/uefi/boot/initramfs.cpio.gz && sudo chown -R $USER:$(id -gn) *

grub-mkrescue -o ${WORKDIR}/build/amd64/tinilinux-amd64-uefi.iso ${WORKDIR}/build/amd64/uefi

https://haoict.github.io/operating-systems/linux/make-your-own-linux-distro-from-scratch/